B-G431B-ESC1 Project Roadmap

UART-Controlled BLDC Motor Node for Autonomous RC Car

Review, Analysis & Improvements

| Motor Parameter | Value / Note |
| --- | --- |
| Motor | 2852-3100 KV (3S LiPo, 2200 mAh) |
| Pole pairs | 2 |
| Stator resistance Rs | 0.1 Ohm |
| Stator inductance Ls | 0.001 mH (note: likely 1 uH range, verify) |
| Peak current Imax | 2 A pk |
| Back-EMF constant Ke | 3.0 Vrms/kRPM |
| Rotor inertia J | 0.09 uN.m.s^2 |
| Friction B | 0.45 uN.m.s |

# 1.  Overall Assessment

Your 7-phase roadmap is well-structured and technically sound. It follows a logical bottom-up sequence: from getting the motor spinning reliably, through robustness improvements, firmware architecture, state machine, communication, bench testing, and finally acceptance criteria. This is exactly the right order for an embedded motor-control project.

The bare-minimum document correctly identifies the true first milestone: a UART-controlled smart actuator that accepts a normalized [-1, 1] command and returns estimated speed safely. That is the right anchor point to keep the project from scope-creep early.

| Verdict | The roadmap is already good. The improvements below are mostly about making each phase clearer, filling in gaps that could block you later, and adding explicit teaching/review hooks since you mentioned this must also be readable as a learning reference. |
| --- | --- |

# 2.  Phase-by-Phase Review

## Phase 1 — Stable ST Baseline

What you have:

- Keep Motor Pilot working

- Verify startup, handover, closed-loop behavior

- Confirm estimated speed is usable

Assessment:

This is the right gate. Nothing above it is valid until the motor starts reliably and the speed estimate is trustworthy. One important clarification missing from the original: Motor Pilot communicates over SWD/ST-Link, not UART — so there is no conflict between keeping ST-Link alive for tuning and using the USB-UART for RPi communication later. Make this explicit in your notes because it is a common confusion point.

Suggested additions:

- Define a clear pass/fail criterion: e.g., 10 consecutive startups without fault at zero load.

- Record a baseline telemetry capture (speed estimate vs. time) from Motor Pilot so you have a reference to compare against after later changes.

- Note which Motor Pilot parameters you are monitoring: Id/Iq references, bus voltage, estimated speed (omega_r or RPM), FOC state machine state.

- Explicitly verify that the sensorless observer locks before 20% of free-spin speed — this is the boundary where BEMF is sufficient for reliable estimation.

## Phase 2 — Sensorless Robustness

What you have:

- Improve rev-up

- Improve observer lock

- Reduce startup failures

- Test light disturbances and light load

Assessment:

Correct focus. For a high-KV motor (3100 KV) on 3S (12.6 V max), the electrical frequency at even modest RPM is high, and the sensorless observer needs careful tuning. With Ls = 0.001 mH (extremely low inductance), the current ripple at typical PWM frequencies will be large, which can inject noise into the observer. This phase should also address that.

Suggested additions:

- Add a note on the observer type in use (PLL-based, Luenberger, or sliding-mode). The STM32G431 MC SDK typically uses a PLL observer with a configurable bandwidth — this bandwidth is the primary tuning knob.

- Add a test for minimum reliable speed: find the lowest RPM at which the speed estimate remains stable under a small hand-applied load.

- Document the rev-up ramp profile (acceleration rate in RPM/s) that you land on — this becomes a parameter in the state machine later.

- Verify current-loop bandwidth is adequate: with Rs=0.1 Ohm and Ls~1 uH, the electrical time constant is tau_e = Ls/Rs = 10 us, meaning your current loop must close well within that.

## Phase 3 — Firmware Architecture Split

What you have:

- Separate motor-control core / application layer / communication layer

- Create two builds: tuning build and RPi deployment build

Assessment:

This is the most important architectural decision in the project and it is under-specified in the current roadmap. The split between a tuning build (ST-Link active, Motor Pilot accessible) and a deployment build (UART to RPi) is essential — but HOW to implement the split needs to be spelled out to avoid rework.

Suggested structure:

- motor_core/: FOC loop, observer, current control — never modified by the application layer. This is the 'plant interface'.

- app/: State machine, command mapping [-1,1] to torque/speed reference, safety logic.

- comms/: UART driver, packet framing, watchdog. In the tuning build this layer is a stub; in the deployment build it is live.

- Use a single compile-time flag (e.g., #define BUILD_MODE TUNING vs DEPLOY) to switch between builds, not separate CubeIDE projects, to reduce maintenance burden.

- Document the API boundary between motor_core and app explicitly: what functions does app call? What callbacks does motor_core provide?

| Teaching note | This phase is the most valuable from a learning perspective. A clean layered architecture with explicit API boundaries is the difference between firmware that can be tested, reviewed, and maintained versus firmware that becomes impossible to understand after 3 months. |
| --- | --- |

## Phase 4 — ESC State Machine

What you have:

- States: BOOT, WAIT_NEUTRAL, READY, FORWARD, BRAKE, REVERSE, FAULT

- Rules: neutral at startup, sign reversal goes to BRAKE first, reverse only near zero, timeout to safe state, fault to safe stop

Assessment:

This is the right state set. One gap: there is no STOPPING state between FORWARD/REVERSE and READY. When the motor is decelerating from a running state back to idle, the system needs a defined transient state — otherwise the transition to BRAKE or READY may be ambiguous, especially when the RPi sends u=0.

Suggested state machine additions:

| State | Entry condition | Exit condition |
| --- | --- | --- |
| BOOT | Power-on reset | Peripherals init OK |
| WAIT_NEUTRAL | After BOOT | u received in [-0.05, +0.05] |
| READY | Neutral confirmed | u outside deadband -> FORWARD/REVERSE |
| FORWARD | u > +deadband | u = 0 -> STOPPING; u < 0 -> BRAKE |
| STOPPING | u = 0 from FORWARD/REVERSE | omega < omega_min -> READY |
| BRAKE | Sign reversal from FORWARD | omega ~ 0 -> READY or REVERSE |
| REVERSE | From BRAKE when speed ~ 0 | u = 0 -> STOPPING; u > 0 -> BRAKE |
| FAULT | Overcurrent, timeout, observer loss | Manual reset or watchdog reboot |

Additional rules to make explicit:

- Define the neutral deadband numerically: suggested |u| < 0.05.

- Define 'near zero speed' for BRAKE->REVERSE: suggested |omega_estimated| < 50 RPM.

- Define the UART watchdog timeout: suggested 100 ms without a valid packet triggers FAULT.

- Define what 'usable' means for the speed estimate: maximum acceptable noise band.

## Phase 5 — UART Protocol with RPi5

What you have:

- Normalized command u in [-1, 1]

- Enable bit / watchdog

- Telemetry: estimated speed, state, voltage, current, faults

Assessment:

Good scope. The protocol design needs to be pinned down more concretely before coding starts — otherwise the RPi-side and STM32-side implementations will drift. A minimal but complete protocol specification should cover: frame format, baud rate, endianness, error detection, and timing.

Recommended minimal protocol (for teaching clarity):

| Direction | Field | Type | Description |
| --- | --- | --- | --- |
| RPi -> STM | header | 0xAA 0x55 | Start-of-frame sync bytes |
|  | cmd_u | int16 (x1000) | [-1000, +1000] maps to [-1.0, +1.0] |
|  | enable | uint8 | 0 = disable motor, 1 = enable |
|  | crc8 | uint8 | CRC-8 of preceding bytes |
| STM -> RPi | header | 0x55 0xAA | Start-of-frame (different from cmd) |
|  | state | uint8 | State machine enum value |
|  | speed_rpm | int16 | Estimated mechanical speed (RPM) |
|  | vbus_mv | uint16 | Bus voltage in mV |
|  | iq_ma | int16 | Q-axis current in mA (proportional to torque) |
|  | fault_flags | uint8 | Bitfield: bit0=overcurrent, bit1=timeout, bit2=observer_loss |
|  | crc8 | uint8 | CRC-8 of preceding bytes |

- Baud rate: 115200 baud is sufficient and universally reliable on both RPi5 UART and STM32G431.

- Suggested cycle rate: RPi sends commands at 50 Hz; STM responds after each received command (request-response pattern). This is simpler to debug than independent asynchronous streams.

- Use DMA + IDLE-line detection on STM32 UART for efficient packet reception without polling.

## Phase 6 — Bench and Drivetrain Testing

What you have:

- Free motor, disturbance by hand, drivetrain off-ground, low-speed rolling, forward/brake/reverse transitions

Assessment:

Good incremental sequence. One important addition: include a test for UART watchdog recovery. Deliberately drop the RPi connection and verify the STM32 transitions to FAULT and stops the motor within the timeout. This is safety-critical and should be a first-class test, not an afterthought.

Suggested test sequence within this phase:

- Test 1 — Baseline free-spin: send u = 0.3, confirm motor spins, confirm telemetry is live.

- Test 2 — Watchdog: send commands for 5 s then stop RPi process; confirm FAULT within timeout.

- Test 3 — Sign reversal: run at u = 0.5, command u = -0.5; confirm BRAKE state before REVERSE.

- Test 4 — Hand load: apply gentle load with gloved hand at running speed; confirm speed estimate responds and Iq rises.

- Test 5 — Drivetrain off-ground: repeat tests 1-3 with wheel spinning freely.

- Test 6 — Low-speed roll: first real surface test at very low speed (u ~ 0.1) on a flat surface.

## Phase 7 — Acceptance Criteria

What you have:

- Startup success rate, speed estimate quality, safe reversals, timeout behavior, real-car usability

Assessment:

These are the right categories but they are not yet measurable. Acceptance criteria must be numeric to be actionable.

Suggested quantified acceptance criteria:

| Criterion | Minimum acceptable | Target |
| --- | --- | --- |
| Startup success rate (10 attempts) | >= 8/10 | 10/10 |
| Speed estimate noise at steady state | +/- 100 RPM | +/- 50 RPM |
| Watchdog fault latency | <= 200 ms | <= 100 ms |
| BRAKE->REVERSE safety (no direct flip) | 100% of attempts | 100% of attempts |
| UART round-trip latency (cmd -> telemetry) | <= 25 ms at 50 Hz | <= 10 ms at 50 Hz |
| Command response linearity (u vs speed) | Monotonic | R^2 > 0.95 over [0.2, 1.0] |

# 3.  Proposed Additions to the Roadmap

## 3.1  Phase 1.5 — Control Loop Verification (new phase)

Between Phase 1 (stable baseline) and Phase 2 (robustness), insert a short verification step for the current-loop and speed-loop controllers. This is justified by your motor parameters:

- With Rs = 0.1 Ohm and Ls ~ 1 uH, the electrical time constant is tau_e = Ls/Rs = 10 us. The current-loop bandwidth set in Motor Pilot should be well below f_sw / 10 (e.g., if PWM is 20 kHz, current loop bandwidth < 2 kHz).

- Verify the speed loop P and I gains are consistent with the identified inertia (J = 0.09 uN.m.s^2) and friction (B = 0.45 uN.m.s). With these values, the mechanical time constant is tau_m = J/B = 0.2 s — a reasonable speed loop bandwidth is 5-10 rad/s.

- A simple step response test (step u from 0 to 0.3) with Motor Pilot scope will show whether the current loop and speed loop are tuned correctly before adding the state machine.

## 3.2  Phase 4.5 — Hardware-in-the-Loop Test with RPi (new phase)

Between Phase 4 (state machine) and Phase 5 (full UART protocol), add a minimal HIL test:

- Write a minimal RPi5 Python script that sends fixed packets and reads back raw bytes over UART.

- Verify byte framing, CRC, and timing before integrating the full protocol.

- This phase catches wiring mistakes, baud rate mismatches, and level-shifting issues (RPi5 is 3.3 V; B-G431B-ESC1 UART is also 3.3 V via the ST-Link USB bridge — confirm this before connecting).

## 3.3  Teaching and Review Artifacts (new cross-cutting concern)

Since this project is also meant to be readable as a learning reference, add these artifacts at the end of each phase:

- Phase summary comment block in the code: a structured comment at the top of each new module explaining what it does, what assumptions it makes, and what the next module above it expects from it.

- State machine diagram: a simple hand-drawn or draw.io diagram of the state machine committed to the repo alongside the code.

- Parameter registry: a single header file (esc_params.h) listing all tunable parameters with their current values and physical units. This makes the project self-documenting.

- Test log template: a simple markdown file in the repo for recording test results per phase. This is extremely useful when returning to the project after a break.

# 4.  Revised Full Roadmap

| Phase | Name | Key deliverable | Gate (pass/fail) |
| --- | --- | --- | --- |
| 1 | Stable ST Baseline | Motor starts reliably, speed estimate visible in Motor Pilot | 10 startups OK |
| 1.5 | Control Loop Verification | Step response recorded, gains confirmed vs. motor parameters | Monotonic step response |
| 2 | Sensorless Robustness | Startup reliability improved, minimum stable speed documented | 8/10 starts OK |
| 3 | Firmware Architecture Split | motor_core / app / comms layers separated, two build configs | Both builds compile and run |
| 4 | ESC State Machine | Full state machine implemented and unit-testable on bench | All transitions verified |
| 4.5 | HIL Framing Test | RPi Python script exchanges raw UART frames with STM32 | No framing errors over 1000 packets |
| 5 | Full UART Protocol | Defined protocol, CRC, watchdog, all telemetry fields live | Watchdog fault < 100 ms |
| 6 | Bench & Drivetrain Testing | All 6 test scenarios completed and logged | All 6 tests pass |
| 7 | Acceptance Criteria | Quantified criteria met, project declared usable | See Section 2 table |

# 5.  Key Technical Notes for Teaching Clarity

## 5.1  Why Two Builds Instead of Runtime Switching?

Motor Pilot communicates over the STM32 MC SDK's internal debug interface (through the ST-Link SWD port). This is separate from the UART. You do NOT need to disable Motor Pilot to run UART. However, once you move to a deployed build on the real car without ST-Link connected, the Motor Pilot variables are simply not accessible — which is fine. The two-build approach keeps the tuning-specific code out of the deployment binary, making the deployed firmware smaller and easier to reason about.

## 5.2  Speed Estimate vs. Speed Control

Your requirement is to output the estimated speed as a telemetry value, not to implement a closed outer speed loop. This is the right choice for the first version: the RPi can close the outer loop in software using the telemetry feedback if needed, or the ESC can operate in torque mode (direct Iq reference from u). Be explicit in the architecture about which mode is in use.

For teaching: the inner FOC loop (current/torque control) is mandatory. An outer speed loop is optional and should only be added in Phase 2 or later if needed for the RC car use case.

## 5.3  Ls Value Check

Your identified Ls = 0.001 mH = 1 uH is extremely low. For a 2852-size motor this is plausible but worth double-checking, since most 28xx-series BLDC motors have Ls in the range of 5-50 uH. If Motor Pilot's identification used 0.001 mH and the current loop was tuned accordingly, the result would be a very high bandwidth current loop. Verify this value with a bench LCR meter if possible, or check whether the Motor Pilot identification converged cleanly.

## 5.4  USB as UART — Practical Notes

The B-G431B-ESC1 exposes a USB CDC (virtual COM port) through the ST-Link. In the deployment build, you can redirect UART1/UART2 to the USB CDC interface so the RPi connects over USB as a serial port. Alternatively, the board has test pads for UART that can be used with a dedicated USB-to-serial adapter. Choose one approach and document it as a project decision, since it affects both the STM32 firmware UART peripheral configuration and the RPi /dev/ttyXXX device path.

Document generated: March 2026  —  B-G431B-ESC1 / STM32G431CB  —  Motor 2852-3100KV